Skill session: Viscosity and biodiesel

This task addresses the following Key knowledge descriptors:

Unit 3 AOS1: the comparison of the suitability of petrodiesel and biodiesel as transport fuels with reference to sources, chemical structures, combustion products, flow along fuel lines (implications of hygroscopic properties and impact of outside temperature on viscosity) and the environmental impacts associated with their extraction and production.

Key science skills

- organise, present and interpret data using schematic diagrams and flow charts, balanced chemical equations, tables, graphs, percentages and calculations of mean

- take a qualitative approach when identifying and analysing experimental data with reference to accuracy, precision, reliability, validity, uncertainty and errors (random and systematic)

- draw conclusions consistent with evidence and relevant to the question under investigation

Outline

As a part of the European Council’s commitment to sourcing 20% of their energy from renewable sources, there has been a push towards the production and use of biofuels for transport. However, for bioidiesel to be used commercially in Europe the fuel must have properties that make it suitable for temperatures which range between      -20 and 35oC. Temperature is known to affect the viscosity of fuels which then affects flow in the fuel lines.

The effect of temperature on the viscosity of biodiesel and biodiesel: petrodiesel blends was investigated in a research study.

The change in viscosity with temperature is shown in the graph below.

Use the information from the graph to answer questions 1 and 2.

- What is the effect of temperature on the viscosity of biodiesel?

- If the appropriate range for the viscosity of diesel is between 4 -5 cSt, what conclusion can you make about the suitability of the biodiesel and biodiesel blends as a commercial fuel in Europe?

- What modifications could be made to the diesel engine to improve the suitability of biodiesel?

- Explain why the viscosity of the fuels differs. In your answer refer to the structures of biodiesel and petrodiesel.

- The viscosity of the fuel was measured by using a high-pressure viscometer which is based on measuring the rate at which a sinker falls in a metal cylinder (Figure 1). As the measurements from this technique are known to be affected by a vortex forming as the sinker falls through the liquid, the equipment is calibrated by using octane which has a known viscosity of 0.65 cSt at 38oC.

  - Will calibration with octane improve the accuracy of the viscosity measurements of the biofuel? Explain.

Figure 1. High pressure viscometer.

Is the effect of the vortex on the viscosity result a systematic or a random error? Explain.

- Write a conclusion for the investigation based on the evidence presented in the graph. Identify any limitations in the conclusion and describe what further studies should be undertaken.