# 12 Chemistry 2.4 Production of Bioethanol

VCE Chemistry Unit 3 - Area of Study 1

## 1. Overview

Bioethanol (C₂H₅OH) is a renewable biofuel produced through fermentation of glucose from plant sources, followed by distillation to increase concentration. It serves as a more sustainable alternative to petrol for transportation.

## 2. The Fermentation Equation

The balanced equation for glucose fermentation is:

C₆H₁₂O₆(aq) → 2C₂H₅OH(aq) + 2CO₂(g)

glucose → ethanol + carbon dioxide

## 3. Production Process

Step 1: Source Selection

- First-generation: Food crops (sugarcane, corn, wheat)

- Second-generation: Agricultural waste, wood chips, grass

- Sugar extraction: Complex carbohydrates (starch, cellulose) broken down to glucose

Step 2: Fermentation

- Catalyst: Yeast (Saccharomyces cerevisiae) – contains enzymes

- Conditions: Anaerobic (absence of oxygen), 25–37°C, pH 4–6

- Duration: 24–72 hours

- Products: Ethanol (~10–15% concentration) + CO₂ (bubbles off)

- Limitation: High ethanol concentration kills yeast

Step 3: Distillation

- Purpose: Separate ethanol from water (different boiling points)

- Method: Heat fermentation mixture; ethanol (b.p. 78°C) evaporates before water (b.p. 100°C)

- Result: Concentrated ethanol (~95% purity) suitable for fuel

- Dehydration: Further processing to 99.5% for blending with petrol

## 4. Sustainability as Transport Fuel

Environmental Benefits:

- Renewable resource: Crops regrown annually

- Carbon cycle: CO₂ released during combustion was recently absorbed during photosynthesis (near carbon-neutral)

- Cleaner burning: Lower particulate emissions than petrol

- Biodegradable: Less environmental damage from spills

Challenges:

- Land use: Competition with food production

- Energy inputs: Growing, harvesting, processing, transport require energy which is often from fossil fuels

- Lower energy density: ~29.7 kJ.g-1 vs ~45 kJ.g-1 for petrol

- Water usage: Crop irrigation requirements

## 5. Applications

- E10 fuel: 10% ethanol, 90% petrol (most common blend)

- E85: 85% ethanol, 15% petrol (flex-fuel vehicles)

- E100: Pure ethanol (some Brazilian vehicles)

- Advantages in engines: Higher octane rating, cleaner combustion

## 6. Key Points

- Fermentation equation: C₆H₁₂O₆(aq) → 2C₂H₅OH(aq) + 2CO₂(g)

- Produced by yeast in anaerobic conditions

- Distillation required to concentrate ethanol for fuel use

- More sustainable than fossil fuels (renewable, lower emissions)

- From glucose in plant sources to usable transport fuel