Topic 2.2: Carbon-Based Fuels

Expanded Knowledge Blocks

## T2.2, KB1: Fuel Definition & Classification

Visual Cue: Timeline from millions of years to annual crops

Definition: A fuel is a substance that can be burned or otherwise reacted to produce energy, typically in the form of heat or electricity. Carbon-based fuels contain carbon and hydrogen atoms.

## Two Main Categories:

### FOSSIL FUELS (Non-renewable):

- Formation time: Millions of years

- Source: Ancient organic matter (geological)

- Cannot be replenished within human timescales

- Adds previously sequestered carbon to atmosphere

- Examples: Coal, natural gas, petrol

### BIOFUELS (Renewable):

- Formation time: Weeks to years

- Source: Recently living organisms (biological)

- Can be continuously produced through natural processes

- Part of active carbon cycle (carbon recently absorbed)

- Examples: Biogas, bioethanol, biodiesel

### Why Renewability Matters:

- Resource availability (finite vs. sustainable)

- Climate change (ancient carbon vs. carbon-neutral cycle)

- Energy security (domestic production possible)

- Waste utilization (circular economy)

# T2.2, KB2: Coal

Visual Cue: Black solid rock, compressed ancient swamp forest layers

### Formation & Composition:

- Formed from compressed plant material over 300+ million years

- Swampy environments under high pressure

- Primarily carbon with varying H, S, O, N

- State: Solid

### Combustion Equation:

C(s) + O₂(g) → CO₂(g)

ΔH = -394 kJ mol⁻¹

### Uses:

- Electricity generation

- Steel production

- Cement manufacturing

### Environmental Impact:

- HIGHEST CO₂ emissions per unit energy

- Produces SO₂ → acid rain

- Particulate matter (air pollution)

- Mercury and heavy metal emissions

# T2.2, KB3: Natural Gas

Visual Cue: Blue flame, underground gas deposit with bubbles

### Formation & Composition:

- Formed from decomposed organic matter under high pressure/temperature over millions of years

- Primary component: Methane (CH₄)

- Smaller amounts: Ethane, propane, butane

- State: Gas (transported as LNG - Liquefied Natural Gas)

### Combustion Equation (Methane):

CH₄(g) + 2O₂(g) → CO₂(g) + 2H₂O(l)

ΔH = -890 kJ mol⁻¹

Heat of combustion: ~54 kJ g⁻¹

### Uses:

- Heating and cooking

- Electricity generation

- Industrial processes

### Environmental Impact:

- Lower CO₂ emissions than coal or petrol per unit energy

- Concern: Methane leakage during extraction/transport

- Methane is a powerful greenhouse gas (25× more potent than CO₂ over 100 years)

# T2.2, KB4: Petrol

Visual Cue: Pink liquid at fuel pump, oil refinery towers

### Formation & Composition:

- Refined from crude oil by fractional distillation

- Crude oil formed from ancient marine organisms over millions of years

- Complex mixture of hydrocarbons: primarily C₇–C₁₁ alkanes and cycloalkanes

- State: Liquid

### Combustion Equation (Octane as representative):

2C₈H₁₈(l) + 25O₂(g) → 16CO₂(g) + 18H₂O(l)

ΔH = -5470 kJ mol⁻¹ (per mole of octane)

Heat of combustion:

  - ~45 kJ g⁻¹

  - ~34 kJ mL⁻¹

### Uses:

- Primary transportation fuel for internal combustion engines

- Portable power generation

### Environmental Impact:

- Significant CO₂ emissions

- Air pollutants: nitrogen oxides (NOₓ), particulates

- Volatile organic compounds (VOCs)

- Ground-level ozone formation

# T2.2, KB5: Biogas

Visual Cue: Bubbling digester tank with organic waste

### Formation & Composition:

- Produced by anaerobic digestion of organic matter

- Sources: Animal waste, food waste, sewage, agricultural residues

- Microorganisms break down material in absence of oxygen

- State: Gas

### Composition:

- 45–75% methane (CH₄)

- 25–55% carbon dioxide (CO₂)

- Trace: H₂S, NH₃, other gases

### Production & Combustion Equations:

Anaerobic digestion (simplified):

C₆H₁₂O₆(aq) → 3CH₄(g) + 3CO₂(g)

Combustion (methane component):

CH₄(g) + 2O₂(g) → CO₂(g) + 2H₂O(l)

ΔH = -890 kJ mol⁻¹

Heat of combustion: 14– 24 kJ g⁻¹ (varies with methane content)

### Renewability:

- Production cycle: Weeks to months

- Organic waste continuously produced

- Sustainable when using waste materials

### Uses:

- Heating and cooking

- Electricity generation

- Vehicle fuel (when purified to bio-methane)

### Environmental Benefits:

- Reduces methane emissions from decomposing waste

- Utilises waste products (circular economy)

- Carbon-neutral cycle (recent carbon)

- Reduces landfill volume

# T2.2, KB6: Bioethanol

Visual Cue: Clear liquid fuel, sugarcane/corn field → factory → distillation tower → transport → E10 pump

## Formation Process (Complete Pathway):

### Step 1 - Photosynthesis (crops grow):

6CO₂(g) + 6H₂O(l) → C₆H₁₂O₆(aq) + 6O₂(g)

- Crops: Sugarcane, corn, wheat absorb atmospheric CO₂

### Step 2 - Fermentation (glucose → ethanol):

C₆H₁₂O₆(aq) → 2C₂H₅OH(aq) + 2CO₂(g)

- Yeast in anaerobic conditions, Temperature: 25–37°C

### Step 3 - Distillation:

- Separates ethanol from water, Ethanol b.p. 78°C vs. water b.p. 100°C

- Concentration: ~95% (further dehydration to 99.5% for fuel)

### Step 4 - Transportation:

- Delivered to fuel stations (often using fossil fuel trucks)

- Mixed with petrol (e.g., E10 = 10% ethanol, 90% petrol)

### Combustion Equation:

C₂H₅OH(l) + 3O₂(g) → 2CO₂(g) + 3H₂O(l)

ΔH = -1370 kJ mol⁻¹

Heat of combustion: ~29.7 kJ g⁻¹

### Renewability:

- Production cycle: Months to one year

- Crops replanted annually

- Chemical formula: C₂H₅OH

### Uses:

- Blended with petrol (E10, E85, E100)

- Transportation fuel

- Can be used pure in adapted engines

# T2.2, KB6: Bioethanol continued

### Environmental Considerations:

Benefits:

  - Lower CO₂ emissions than petrol (carbon absorbed during growth)

  - Part of active carbon cycle

  - Biodegradable

Challenges:

  - Land use competition with food crops

  - Energy inputs for production (fertilizers, processing, distillation)

  - Water usage for irrigation

  - Transport emissions (if using fossil fuel vehicles)

# T2.2, KB7: Biodiesel

Visual Cue: Yellow-green liquid, oil seed crops (canola/soy/palm), transesterification reactor

### Formation & Composition:

- Produced by transesterification of plant oils or animal fats

- Sources: Soybean, canola, palm oil, animal fats

- Reacted with alcohols (typically methanol or ethanol)

- Products: Fatty acid methyl esters (FAMEs) or fatty acid ethyl esters (FAEEs)

- State: Liquid

### Transesterification Reaction:

General process:

Triglyceride + 3CH₃OH → 3 FAME + Glycerol

- Catalyst: Usually NaOH or KOH

- Temperature: 60–80°C

### Combustion:

Heat of combustion: ~37 kJ g⁻¹

### Renewability:

- Production cycle: Months to one year

- Oil crops replanted annually

### Uses:

- Diesel engine fuel

- Can be blended with petroleum diesel (B5, B20, B100)

- Can be used pure in adapted engines

### Environmental Considerations:

Benefits:

  - Lower particulate emissions than petroleum diesel

  - Biodegradable

  - Non-toxic

  - Part of active carbon cycle

Challenges:

  - Land use for oil crops (food vs. fuel debate)

  - Deforestation concerns (especially palm oil)

  - Lower energy density than petroleum diesel

  - May require engine modifications for high blends

# T2.2, KB8: Key Comparison Table

Visual Cue: Side-by-side timeline comparison chart

| Characteristic | Fossil Fuels | Biofuels |
| --- | --- | --- |
| Formation time | Millions of years | Weeks to years |
| Renewability | Non-renewable (finite) | Renewable (continuous) |
| Source | Ancient organic matter (geological) | Recently living organisms (biological) |
| Carbon cycle | Adds previously sequestered carbon | Part of active carbon cycle |
| Examples | Coal, natural gas, petrol | Biogas, bioethanol, biodiesel |
| Energy security | Import dependent (many countries) | Domestic production possible |
| Infrastructure | Well-established | Developing |

# T2.2, KB9: Important Sustainability Equations Summary

Visual Cue: Circular diagram showing carbon cycle connections

### Photosynthesis (Carbon capture):

6CO₂(g) + 6H₂O(l) → C₆H₁₂O₆(aq) + 6O₂(g)

### Cellular Respiration (Energy release):

C₆H₁₂O₆(aq) + 6O₂(g) → 6CO₂(g) + 6H₂O(l)

### Fermentation (Bioethanol production):

C₆H₁₂O₆(aq) → 2C₂H₅OH(aq) + 2CO₂(g)

### Complete Combustion Examples:

Methane:

CH₄(g) + 2O₂(g) → CO₂(g) + 2H₂O(l)

Ethanol:

C₂H₅OH(l) + 3O₂(g) → 2CO₂(g) + 3H₂O(l)

Octane:

2C₈H₁₈(l) + 25O₂(g) → 16CO₂(g) + 18H₂O(l)

# T2.2, KB10: Biofuel Challenges

Visual Cue: Balance scale weighing benefits vs. challenges

### Key Challenges:

- Land use: Competition with food crops ("food vs. fuel")

- Energy inputs: Fertilizers, processing, transportation

- Lower energy density: Compared to many fossil fuels

- Infrastructure: Requires investment for large-scale adoption

- Water usage: Irrigation requirements for crops

- Deforestation: Risk with certain crops (e.g., palm oil)

### Why Biofuels Still Matter Despite Challenges:

- Resource availability: Finite fossil fuels will be depleted

- Climate change: Closer to carbon-neutral cycle

- Energy security: Domestic production reduces import dependence

- Waste utilization: Some biofuels (biogas) use waste products

- Technological improvement: Ongoing research improving efficiency