Topic 2.4: Bioethanol

Expanded Knowledge Blocks

## T2.4, KB1: Bioethanol Overview & Definition

Visual Cue: sugarcane field  fermentation vessel distillation  transport  E10 fuel

Definition: Renewable biofuel (C₂H₅OH) produced through fermentation of glucose from plant sources

### Key Characteristics:

- Renewable resource from plant biomass

- More sustainable alternative to petrol

- Used as transport fuel after concentration

- Produced through fermentation + distillation

## T2.4, KB2: Fermentation Process

Visual Cue: Bubbling fermentation vessel, yeast cells, CO₂ bubbles

### Fermentation Equation:

C₆H₁₂O₆(aq) → 2C₂H₅OH(aq) + 2CO₂(g)

glucose → ethanol + carbon dioxide

### Process Requirements:

Catalyst: Yeast (Saccharomyces cerevisiae) – contains enzymes

Conditions: Anaerobic (absence of oxygen), 25–37°C, pH 4–6

Duration: 24–72 hours

Products: Ethanol (~10–15% concentration) + CO₂ (bubbles off)

Limitation: High ethanol concentration kills yeast

## T2.4, KB3: Source Selection & Preparation

Visual Cue: Sugarcane/corn crops in a field and pile of wood chips

### Feedstock Options:

First-generation biofuels:

- Food crops: sugarcane, corn, wheat

- High sugar or starch content

- Easier to process but competes with food production

Second-generation biofuels:

- Agricultural waste, wood chips, grass

- Cellulose-based materials

- More sustainable but requires more processing

### Sugar Extraction:

- Complex carbohydrates (starch, cellulose) broken down to glucose

- Enzymatic or chemical processes used

## T2.4, KB4: Distillation

Visual Cue: Distillation column, vapour rising, liquid condensing

Purpose: Separate ethanol from water based on different boiling points

### Process:

- Heat fermentation mixture

- Ethanol (b.p. 78°C) evaporates before water (b.p. 100°C)

- Vapour collected and condensed

- Result: Concentrated ethanol (~95% purity)

### Further Processing:

- Dehydration to 99.5% purity for blending with petrol

- Essential for use as transport fuel

## T2.4, KB5: Sustainability of Bioethanol

Visual Cue: Carbon cycle diagram

### Environmental Benefits:

- Renewable resource: Crops regrown annually

- Carbon cycle: CO₂ released during combustion was recently absorbed during photosynthesis (near carbon-neutral)

- Cleaner burning: Lower particulate emissions than petrol

- Biodegradable: Less environmental damage from spills

### Challenges:

- Land use: Competition with food production

- Energy inputs: Growing, harvesting, processing, transport require energy (mostly from fossil fuels)

- Lower energy density: ~29.7 kJ g⁻¹ vs ~45 kJ g⁻¹ for petrol

- Water usage: Crop irrigation requirements

## T2.4, KB6: Bioethanol Applications

Visual Cue: Fuel pumps showing E10, E85, E100  labels

### Fuel Blends:

E10 fuel:

- 10% ethanol, 90% petrol

- Most common blend in standard vehicles

E85:

- 85% ethanol, 15% petrol

- Requires flex-fuel vehicles

E100:

- Pure ethanol

- Used in some Brazilian vehicles

### Engine Advantages:

- Higher octane rating than petrol

- Cleaner combustion with reduced emissions

- Compatible with modern engine technology

## T2.4, KB7: Key Summary Points

Visual Cue: Flow diagram from crop → glucose → fermentation → distillation → fuel

### Essential Knowledge:

- Fermentation equation: C₆H₁₂O₆(aq) → 2C₂H₅OH(aq) + 2CO₂(g)

- Produced by yeast in anaerobic conditions

- Distillation required to concentrate ethanol for fuel use

- More sustainable than fossil fuels (renewable, lower emissions)

- Process: Glucose from plant sources → fermentation → distillation → usable transport fuel

### Critical Comparisons:

- Heat of combustion: ~29.7 kJ g⁻¹ (bioethanol) vs ~45 kJ g⁻¹ (petrol)

- Near carbon-neutral due to recent photosynthesis

- Balance: renewable resource vs land/water requirements